Central Connecticut State University

BUILDING  A PIPELINED DATAPATH WITH 5-STAGES (IF,ID,EX, MEM,WB)

Final Project

prepared by

Thi Minh Huyen Le

Brendan Manzolli

Erik Marrero

Fall - 2021

- Tasks:

Thi Minh Huyen Le: Combine Code. Results

Brendan Manzolli: Multiplexes, Diagrams

Erik Marrero: MainControl, Instructions

- Description

Instructions:

| Instruction (I-Type: addi) | Opcode (4 bits) [15-12] | Rs (2 bits) [11-10] | Rt (2 bits) [9-8] | Value or Address (8 bits) [7-0] | Binary Instruction |
| --- | --- | --- | --- | --- | --- |
| nop | 0000 | 00 | 00 | 00000000 | 0000_00_00_00000000 |
| lw $1, 0($0) | 1001 | 00 | 01 | 00000000 | 1001000100000000 |
| lw $2, 2($0) | 1001 | 00 | 10 | 00000010 | 1001001000000010 |
| beg $3,$0,IMemory[15] | 1011 | 00 | 11 | 00000101 | 1011001100000101 |
| sw $1,2($0) | 1010 | 00 | 01 | 00000010 | 1010000100000010 |
| sw $2,0($0) | 1010 | 00 | 10 | 00000000 | 1010001000000000 |
| lw $1,0($0) | 1001 | 00 | 01 | 00000000 | 1001000100000000 |
| lw $2,2($0) | 1001 | 00 | 10 | 00000010 | 1001001000000010 |
| bne $3,$0,IMemory[6] | 1100 | 00 | 11 | 00000010 | 1100110000000010 |

| Instruction (R-Type) | Opcode (4 bits) [15-12] | Rs (2 bits) [11-10] | Rt (2bit) [9-8] | Rd (2bits) [7-6] | Unused (6 bits) [5-0] | Binary Instruction |
| --- | --- | --- | --- | --- | --- | --- |
| slt $3, $1, $2 | 0100 | 01 | 10 | 11 | 000000 | 0100011011000000 |
| sub $3, $1, $2 | 0001 | 01 | 10 | 11 | 000000 | 0001011011000000 |

- Block logic diagrams

CPU :

1-bit ALU for bits 0-14

1-bit ALU for bit 15

16bit-ALU

2x1 Multiplexer

4x1 Multiplexer

The truth table for MainControl

| Instruction | OpCode | RegDst | Beg | Bne | MemtoReg | MemWrite | ALUSrc | RegWrite | ALUCtl |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| add | 0000 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 010 |
| sub | 0001 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 110 |
| and | 0010 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 000 |
| or | 0011 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 001 |
| slt | 0100 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 111 |
| addi | 0101 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 010 |
| slti | 0110 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 111 |
| andi | 0111 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 000 |
| ori | 1000 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 001 |
| lw | 1001 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 010 |
| sw | 1010 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 010 |
| beg | 1011 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 110 |
| bne | 1100 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 110 |

Verilog source code

// Behavioral model of MIPS - pipelined implementation // --------------REGISTER FILE --------------- // Assign RR1, RR2 to RD1, RD2 // When RegWrite is on, assign WD to WR module reg_file (RR1,RR2,WR,WD,RegWrite,RD1,RD2,clock); input [1:0] RR1,RR2,WR; input [15:0] WD; input RegWrite,clock; output [15:0] RD1,RD2; reg [15:0] Regs[0:3]; assign RD1 = Regs[RR1]; assign RD2 = Regs[RR2]; initial Regs[0] = 0; always @(negedge clock) if (RegWrite==1 & WR!=0) Regs[WR] <= WD; endmodule //-----------END REGISTER FILE----------- //--------------ALU---------------------- // A 32-bit ALU constructed from the 31 copies of the 1-bit ALU in the top and one 1-bit ALUmsb in the bottom. module alu (op,a,b,result,zero); input [15:0] a; input [15:0] b; input [2:0] op; output [15:0] result; output zero; wire c1,c2,c3,c4,c5,c6,c7,c8,c9,c10,c11,c12,c13,c14,c15,c16; ALU1 alu0 (a[0],b[0],op[2],op[1:0],set, op[2], c1,result[0]); ALU1 alu1 (a[1],b[1],op[2],op[1:0],1'b0, c1, c2,result[1]); ALU1 alu2 (a[2],b[2],op[2],op[1:0],1'b0, c2, c3,result[2]); ALU1 alu3 (a[3],b[3],op[2],op[1:0],1'b0, c3, c4,result[3]); ALU1 alu4 (a[4],b[4],op[2],op[1:0],1'b0, c4, c5,result[4]); ALU1 alu5 (a[5],b[5],op[2],op[1:0],1'b0, c5, c6,result[5]); ALU1 alu6 (a[6],b[6],op[2],op[1:0],1'b0, c6, c7,result[6]); ALU1 alu7 (a[7],b[7],op[2],op[1:0],1'b0, c7, c8,result[7]); ALU1 alu8 (a[8],b[8],op[2],op[1:0],1'b0, c8, c9,result[8]); ALU1 alu9 (a[9],b[9],op[2],op[1:0],1'b0, c9, c10,result[9]); ALU1 alu10 (a[10],b[10],op[2],op[1:0],1'b0, c10, c11,result[10]); ALU1 alu11 (a[11],b[11],op[2],op[1:0],1'b0, c11, c12,result[11]); ALU1 alu12 (a[12],b[12],op[2],op[1:0],1'b0, c12, c13,result[12]); ALU1 alu13 (a[13],b[13],op[2],op[1:0],1'b0, c13, c14,result[13]); ALU1 alu14 (a[14],b[14],op[2],op[1:0],1'b0, c14, c15,result[14]); ALUmsb alu15 (a[15],b[15],op[2],op[1:0],1'b0, c15, c16,result[15],set); or or1(or01, result[0], result[1], result[2], result[3], result[4], result[5], result[6], result[7], result[8], result[9], result[10], result[11], result[12], result[13], result[14], result[15]); not not1(zero, or01); endmodule // 1-bit ALU for bits 0-2 // A 1-bit ALU that performs AND, OR, addition , subtraction on a and b, module ALU1 (a,b,binvert,op,less,carryin,carryout,result); input a,b,less,carryin,binvert; input [1:0] op; output carryout,result; wire sum, a_and_b, a_or_b, b_inv; not not1(b_inv, b); mux2x1 mux1(b,b_inv,binvert,b1); and and1(a_and_b, a, b); or or1(a_or_b, a, b); fulladder adder1(sum,carryout,a,b1,carryin); mux4x1 mux2(a_and_b,a_or_b,sum,less,op[1:0],result); endmodule // 1-bit ALU for the most significant bit // A 1-bit ALU that performs AND, OR, addition , subtraction on a and b, GIVE THE MOST SIGNIFICANT BIT AS AN OUTPUT SUM module ALUmsb (a,b,binvert,op,less,carryin,overflow,result,sum); input a,b,less,carryin,binvert; input [1:0] op; output overflow,result,sum; wire sum, a_and_b, a_or_b, b_inv; not not1(b_inv, b); mux2x1 mux1(b,b_inv,binvert,b1); and and1(a_and_b, a, b); or or1(a_or_b, a, b); fulladder adder1(sum,overflow,a,b1,carryin); mux4x1 mux2(a_and_b,a_or_b,sum,less,op[1:0],result); endmodule module halfadder (S,C,x,y); input x,y; output S,C; xor (S,x,y); and (C,x,y); endmodule module fulladder (S,C,x,y,z); input x,y,z; output S,C; wire S1,D1,D2; halfadder HA1 (S1,D1,x,y), HA2 (S,D2,S1,z); or g1(C,D2,D1); endmodule // Multiplexor two 1bit-inputs module mux2x1(A,B,select,OUT); input A,B,select; output OUT; wire wire0, wire1; not not1 (notS,select); and and1 (wire0, A,notS), and2 (wire1, B,select); or or1 (OUT,wire0,wire1); endmodule // Multiplexor four 1bit-inputs module mux4x1(i0,i1,i2,i3,select,y); output y; // Output input i0, i1, i2, i3; // Input ports. input [1:0] select; // Select lines. // intermediate wires wire inv0, inv1; // Inverter outputs. wire a0, a1, a2, a3; // AND gates outputs. // Inverters. not not_1 (inv1, select[1]); not not_0 (inv0, select[0]); // 4-input AND gates. and and_0 (a0, i0, inv1, inv0); and and_1 (a1, i1, inv1, select[0]); and and_2 (a2, i2, select[1], inv0); and and_3 (a3, i3, select[1], select[0]); // 4-input OR gate. or or_0 (y, a0, a1, a2, a3); endmodule //--------------END ALU----------------- //--------------MULTIPLEXORS------------- // Multiplexor two 16-bit inputs. module mux16bitx2x1 (A,B,Select,Out); input [15:0] A,B; input Select; output [15:0] Out; mux2x1 m0 (A[0],B[0],Select,Out[0]), m1 (A[1],B[1],Select,Out[1]), m2 (A[2],B[2],Select,Out[2]), m3 (A[3],B[3],Select,Out[3]), m4 (A[4],B[4],Select,Out[4]), m5 (A[5],B[5],Select,Out[5]), m6 (A[6],B[6],Select,Out[6]), m7 (A[7],B[7],Select,Out[7]), m8 (A[8],B[8],Select,Out[8]), m9 (A[9],B[9],Select,Out[9]), m10 (A[10],B[10],Select,Out[10]), m11 (A[11],B[11],Select,Out[11]), m12 (A[12],B[12],Select,Out[12]), m13 (A[13],B[13],Select,Out[13]), m14 (A[14],B[14],Select,Out[14]), m15 (A[15],B[15],Select,Out[15]); endmodule // Multiplexor two 2-bit inputs. module mux2bitx2x1 (A,B,Select,Out); input [1:0] A,B; input Select; output [1:0] Out; mux2x1 m0 (A[0],B[0],Select,Out[0]), m1 (A[1],B[1],Select,Out[1]); endmodule //--------------END MULTIPLEXORS------------ //--------------MAIN CONTROL--------------- /*** 16-bit CPU control source code ***/ module mainCtrl (op, ctrl); input [3:0] op; output reg [9:0] ctrl; // ctrl bits: RegDst, Beg, Bne, MemtoReg, MemWrite, ALUSrc, RegWrite, ALUCtl(3bits) always @(op) case (op) 4'b0000: ctrl <= 10'b1000_001_010; // ADD 4'b0001: ctrl <= 10'b1000_001_110; // SUB 4'b0010: ctrl <= 10'b1000_001_000; // AND 4'b0011: ctrl <= 10'b1000_001_001; // OR 4'b0100: ctrl <= 10'b1000_001_111; // SLT 4'b0101: ctrl <= 10'b0001_011_010; // ADDI 4'b0110: ctrl <= 10'b0001_011_111; // SLTI 4'b0111: ctrl <= 10'b0001_011_000; // ANDI 4'b1000: ctrl <= 10'b0001_011_001; // ORI 4'b1001: ctrl <= 10'b0001_011_010; // LW 4'b1010: ctrl <= 10'b0001_110_010; // SW 4'b1011: ctrl <= 10'b0100_000_110; // BEG 4'b1100: ctrl <= 10'b0010_000_110; // BNE endcase endmodule //------------END MAIN CONTROL---------------- //------------BRANCH CONTROL--------------- module BranchLogic(Zero,Beg,Bne,Branch); input Zero,Beg,Bne; output Branch; wire invZero; not not1 (invZero,Zero); and a1 (a1,Zero,Beg), a2 (a2,invZero,Bne); or or1 (Branch,a1,a2); endmodule //------------END BRANCH CONTROL--------------- module CPU (clock,PC,IFID_IR,IDEX_IR,EXMEM_IR,MEMWB_IR,WD); input clock; output [15:0] PC,IFID_IR,IDEX_IR,EXMEM_IR,MEMWB_IR,WD; initial begin // Program: swap memory cells (if needed) and compute absolute value ¦5-7¦=2 // // BEG WITH NO-OP -------------------------------------- // IMemory[0] = 16'b1001_00_01_00000000; // lw $1, 0($0) $1 <= DM[0] = 5 // IMemory[1] = 16'b1001_00_10_00000010; // lw $2, 2($0) $2 <= DM[1] = 7 // IMemory[2] = 16'b0000_00_00_00000000; // nop // IMemory[3] = 16'b0000_00_00_00000000; // nop // IMemory[4] = 16'b0000_00_00_00000000; // nop // IMemory[5] = 16'b0100_01_10_11_000000; // slt $3, $1, $2 $1 < $2 => $3 =1 // IMemory[6] = 16'b0000_00_00_00000000; // nop // IMemory[7] = 16'b0000_00_00_00000000; // nop // IMemory[8] = 16'b0000_00_00_00000000; // nop // IMemory[9] = 16'b1011_11_00_00000101; // beq $3, $0, IMemory[15] $3 != $0 => continue to SWAP, don't JUMP // IMemory[10] = 16'b0000_00_00_00000000; // nop // IMemory[11] = 16'b0000_00_00_00000000; // nop // IMemory[12] = 16'b0000_00_00_00000000; // nop // IMemory[13] = 16'b1010_00_01_00000010; // sw $1, 2($0) DM[1] <= $1 = 5 // IMemory[14] = 16'b1010_00_10_00000000; // sw $2, 0($0) DM[0] <= $2 = 7 // IMemory[15] = 16'b0000_00_00_00000000; // nop // IMemory[16] = 16'b0000_00_00_00000000; // nop // IMemory[17] = 16'b0000_00_00_00000000; // nop // IMemory[18] = 16'b1001_00_01_00000000; // lw $1, 0($0) $1 <= DM[0] = 7 // IMemory[19] = 16'b1001_00_10_00000010; // lw $2, 2($0) $2 <= DM[1] = 5 // IMemory[20] = 16'b0000_00_00_00000000; // nop // IMemory[21] = 16'b0000_00_00_00000000; // nop // IMemory[22] = 16'b0000_00_00_00000000; // nop // IMemory[23] = 16'b0001_01_10_11_000000; // sub $3, $1, $2 $3 = $1 -$2 = 7-5 =2 // // BNE WITH NO-OP ---------------------------------------- IMemory[0] = 16'b1001_00_01_00000000; // lw $1, 0($0) $1 <= DM[0] = 5 IMemory[1] = 16'b1001_00_10_00000010; // lw $2, 2($0) $2 <= DM[1] = 7 IMemory[2] = 16'b0000_00_00_00000000; // nop IMemory[3] = 16'b0000_00_00_00000000; // nop IMemory[4] = 16'b0000_00_00_00000000; // nop IMemory[5] = 16'b0100_01_10_11_000000; // slt $3, $1, $2 $1 < $2 => $3 = 1 IMemory[6] = 16'b0000_00_00_00000000; // nop IMemory[7] = 16'b0000_00_00_00000000; // nop IMemory[8] = 16'b0000_00_00_00000000; // nop IMemory[9] = 16'b1100_11_00_00000101; // bne $3, $0, IMemory[15] $3 != $0 => continue to JUMP, skip swap IMemory[10] = 16'b0000_00_00_00000000; // nop IMemory[11] = 16'b0000_00_00_00000000; // nop IMemory[12] = 16'b0000_00_00_00000000; // nop IMemory[13] = 16'b1010_00_01_00000010; // sw $1, 2($0) IMemory[14] = 16'b1010_00_10_00000000; // sw $2, 0($0) IMemory[15] = 16'b0000_00_00_00000000; // nop IMemory[16] = 16'b0000_00_00_00000000; // nop IMemory[17] = 16'b0000_00_00_00000000; // nop IMemory[18] = 16'b1001_00_01_00000000; // lw $1, 0($0) $1 <= DM[0] = 5 IMemory[19] = 16'b1001_00_10_00000010; // lw $2, 2($0) $2 <= DM[1] = 7 IMemory[20] = 16'b0000_00_00_00000000; // nop IMemory[21] = 16'b0000_00_00_00000000; // nop IMemory[22] = 16'b0000_00_00_00000000; // nop IMemory[23] = 16'b0001_01_10_11_000000; // sub $3, $1, $2 $3 = $1 -$2 = 5-7 = -2 // // BEG WITHOUT NO-OP -------------------------------------- // IMemory[0] = 16'b1001_00_01_00000000; // lw $1, 0($0) $1 <= DM[0] = 5 // IMemory[1] = 16'b1001_00_10_00000010; // lw $2, 2($0) $2 <= DM[1] = 7 // IMemory[2] = 16'b0100_01_10_11_000000; // slt $3, $1, $2 $1 < $2 => $3 =1 // IMemory[3] = 16'b1011_11_00_0000010; // beq $3, $0, IMemory[6] $3 != $0 => continue to SWAP // IMemory[4] = 16'b1010_00_01_00000010; // sw $1, 2($0) DM[1] <= $1 = 5 // IMemory[5] = 16'b1010_00_10_00000000; // sw $2, 0($0) DM[0] <= $2 = 7 // IMemory[6] = 16'b1001_00_01_00000000; // lw $1, 0($0) $1 <= DM[0] = 7 // IMemory[7] = 16'b1001_00_10_00000010; // lw $2, 2($0) $2 <= DM[1] = 5 // IMemory[8] = 16'b0001_01_10_11_000000; // sub $3, $1, $2 $3 = $1 -$2 = 7-5 = 2 // // BNE WITHOUT NO-OP ---------------------------------------- // IMemory[0] = 16'b1001_00_01_00000000; // lw $1, 0($0) $1 <= DM[0] = 5 // IMemory[1] = 16'b1001_00_10_00000010; // lw $2, 2($0) $2 <= DM[1] = 7 // IMemory[2] = 16'b0100_01_10_11_000000; // slt $3, $1, $2 $1 < $2 => $3 =1 // IMemory[3] = 16'b1100_11_00_00000010; // bne $3, $0, IMemory[6] $3 != $0 => continue to JUMP, skip swap // IMemory[4] = 16'b1010_00_01_00000010; // sw $1, 2($0) // IMemory[5] = 16'b1010_00_10_00000000; // sw $2, 0($0) // IMemory[6] = 16'b1001_00_01_00000000; // lw $1, 0($0) $1 <= DM[0] = 5 // IMemory[7] = 16'b1001_00_10_00000010; // lw $2, 2($0) $2 <= DM[1] = 7 // IMemory[8] = 16'b0001_01_10_11_000000; // sub $3, $1, $2 $3 = $1 -$2 = 5-7 = -2 // Data DMemory [0] = 16'b0101; // switch the cells and see how the simulation output changes DMemory [1] = 16'b0111; // (beq is taken if [0]=32'h7; [1]=32'h5, not taken otherwise) end // Pipeline // IF wire [15:0] PCplus2, NextPC; reg [15:0] EXMEM_Target; wire Branch; reg[15:0] PC, IMemory[0:1023], IFID_IR, IFID_PCplus2; alu fetch (3'b010,PC,16'b10,PCplus2,Unused1); mux16bitx2x1 muxBranch (PCplus2,EXMEM_Target,Branch,NextPC); // ID wire [9:0] Control; reg IDEX_RegWrite,IDEX_MemtoReg, IDEX_Beq, IDEX_Bne, IDEX_MemWrite, IDEX_ALUSrc, IDEX_RegDst; reg [2:0] IDEX_ALUctl; wire [15:0] RD1,RD2,SignExtend, WD; reg [15:0] IDEX_PCplus2,IDEX_RD1,IDEX_RD2,IDEX_SignExt,IDEXE_IR; reg [15:0] IDEX_IR; // For monitoring the pipeline reg [1:0] IDEX_rt,IDEX_rd; reg MEMWB_RegWrite; // part of MEM stage, but declared here before use (to avoid error) reg [1:0] MEMWB_rd; // part of MEM stage, but declared here before use (to avoid error) reg_file rf (IFID_IR[11:10],IFID_IR[9:8],MEMWB_rd,WD,MEMWB_RegWrite,RD1,RD2,clock); mainCtrl MainCtr (IFID_IR[15:12],Control); assign SignExtend = {{8{IFID_IR[7]}},IFID_IR[7:0]}; // EXE reg EXMEM_RegWrite,EXMEM_MemtoReg, EXMEM_Beq, EXMEM_Bne, EXMEM_MemWrite; wire [15:0] Target; reg EXMEM_Zero; reg [15:0] EXMEM_ALUOut,EXMEM_RD2; reg [15:0] EXMEM_IR; // For monitoring the pipeline reg [1:0] EXMEM_rd; wire [15:0] B,ALUOut; wire [1:0] WR; alu branch (3'b010,IDEX_SignExt<<1,IDEX_PCplus2,Target,Unused2); alu ex (IDEX_ALUctl, IDEX_RD1, B, ALUOut, Zero); mux16bitx2x1 muxB (IDEX_RD2, IDEX_SignExt, IDEX_ALUSrc, B); // ALUSrc Mux mux2bitx2x1 muxWR (IDEX_rt, IDEX_rd, IDEX_RegDst, WR); // RegDst Mux // MEM reg MEMWB_MemtoReg; reg [15:0] DMemory[0:1023],MEMWB_MemOut,MEMWB_ALUOut; reg [15:0] MEMWB_IR; // For monitoring the pipeline wire [15:0] MemOut; assign MemOut = DMemory[EXMEM_ALUOut>>1]; always @(negedge clock) if (EXMEM_MemWrite) DMemory[EXMEM_ALUOut>>1] <= EXMEM_RD2; BranchLogic bra (EXMEM_Zero,EXMEM_Beq,EXMEM_Bne,Branch); // notice // WB mux16bitx2x1 muxWD (MEMWB_ALUOut, MEMWB_MemOut, MEMWB_MemtoReg, WD); // MemtoReg Mux //assign WD = (MEMWB_MemtoReg) ? MEMWB_MemOut: MEMWB_ALUOut; // MemtoReg Mux initial begin PC = 0; // Initialize pipeline registers IDEX_RegWrite=0;IDEX_MemtoReg=0;IDEX_Beq=0;IDEX_Bne=0;IDEX_MemWrite=0;IDEX_ALUSrc=0;IDEX_RegDst=0;//IDEX_ALUOp=0; IFID_IR=0; EXMEM_RegWrite=0;EXMEM_MemtoReg=0;EXMEM_Beq=0;EXMEM_Bne=0;EXMEM_MemWrite=0; EXMEM_Target=0; MEMWB_RegWrite=0;MEMWB_MemtoReg=0; end // Running the pipeline always @(negedge clock) begin // IF PC <= NextPC; IFID_PCplus2 <= PCplus2; IFID_IR <= IMemory[PC>>1]; // ID IDEX_IR <= IFID_IR; // For monitoring the pipeline {IDEX_RegDst,IDEX_Beq,IDEX_Bne,IDEX_MemtoReg,IDEX_MemWrite,IDEX_ALUSrc,IDEX_RegWrite,IDEX_ALUctl} <= Control; IDEX_PCplus2 <= IFID_PCplus2; IDEX_RD1 <= RD1; IDEX_RD2 <= RD2; IDEX_SignExt <= SignExtend; IDEX_rt <= IFID_IR[9:8];// change from 11:10 IDEX_rd <= IFID_IR[7:6];// change from 9:8 // EXE EXMEM_IR <= IDEX_IR; // For monitoring the pipeline EXMEM_RegWrite <= IDEX_RegWrite; EXMEM_MemtoReg <= IDEX_MemtoReg; EXMEM_Beq <= IDEX_Beq; EXMEM_Bne <= IDEX_Bne; EXMEM_MemWrite <= IDEX_MemWrite; EXMEM_Target <= Target; EXMEM_Zero <= Zero; EXMEM_ALUOut <= ALUOut; EXMEM_RD2 <= IDEX_RD2; EXMEM_rd <= WR; // MEM MEMWB_IR <= EXMEM_IR; // For monitoring the pipeline MEMWB_RegWrite <= EXMEM_RegWrite; MEMWB_MemtoReg <= EXMEM_MemtoReg; MEMWB_MemOut <= MemOut; MEMWB_ALUOut <= EXMEM_ALUOut; MEMWB_rd <= EXMEM_rd; // WB // Register write happens on neg edge of the clock (if MEMWB_RegWrite is asserted) end endmodule // Test module module test (); reg clock; wire [15:0] PC,IFID_IR,IDEX_IR,EXMEM_IR,MEMWB_IR,WD; CPU test_cpu(clock,PC,IFID_IR,IDEX_IR,EXMEM_IR,MEMWB_IR,WD); always #1 clock = ~clock; initial begin $display ("time PC IFID_IR IDEX_IR EXMEM_IR MEMWB_IR WD"); $monitor ("%2d %3d %b %b %b %b %d", $time,PC,IFID_IR,IDEX_IR,EXMEM_IR,MEMWB_IR,WD); clock = 1; #56 $finish; end endmodule /* Output: time PC IFID_IR IDEX_IR EXMEM_IR MEMWB_IR WD 0 0 00000000 xxxxxxxx xxxxxxxx xxxxxxxx x 1 4 8c010000 00000000 xxxxxxxx xxxxxxxx x 3 8 8c020004 8c010000 00000000 xxxxxxxx x 5 12 00000000 8c020004 8c010000 00000000 0 7 16 00000000 00000000 8c020004 8c010000 5 9 20 00000000 00000000 00000000 8c020004 7 11 24 0022182a 00000000 00000000 00000000 0 13 28 00000000 0022182a 00000000 00000000 0 15 32 00000000 00000000 0022182a 00000000 0 17 36 00000000 00000000 00000000 0022182a 1 19 40 10600005 00000000 00000000 00000000 0 21 44 00000000 10600005 00000000 00000000 0 23 48 00000000 00000000 10600005 00000000 0 25 52 00000000 00000000 00000000 10600005 1 27 56 ac010004 00000000 00000000 00000000 0 29 60 ac020000 ac010004 00000000 00000000 0 31 64 00000000 ac020000 ac010004 00000000 0 33 68 00000000 00000000 ac020000 ac010004 4 35 72 00000000 00000000 00000000 ac020000 0 37 76 8c010000 00000000 00000000 00000000 0 39 80 8c020004 8c010000 00000000 00000000 0 41 84 00000000 8c020004 8c010000 00000000 0 43 88 00000000 00000000 8c020004 8c010000 7 45 92 00000000 00000000 00000000 8c020004 5 47 96 00221822 00000000 00000000 00000000 0 49 100 xxxxxxxx 00221822 00000000 00000000 0 51 104 xxxxxxxx xxxxxxxx 00221822 00000000 0 53 108 xxxxxxxx xxxxxxxx xxxxxxxx 00221822 2 55 112 xxxxxxxx xxxxxxxx xxxxxxxx xxxxxxxx X */

- Test result

Sample Test 1 : DMemory[0] = 5  	DMemory[1] = 7 with Beg instruction

lw $1, 0($0) => $1 = 5

lw $2, 2($0)=> $2 = 7

slt $3, $1, $2=> $3 =1

beg $3, $0, IMemory[6]=> branch does not take

sw $1, 2($0)  => 2($0)=7

sw $2, 0($0)  => 0($0)=5

lw $1, 0($0)  => $1=5

lw $2, 2($0)  => $2=7

sub $3, $1, $2 => $3 = 7-5 = 2 in binary 0000_0000_0000_0010

Sample Test 2 : DMemory[0] = 7  	DMemory[1] = 5 with Beg instruction

lw $1, 0($0) => $1 = 7

lw $2, 2($0)=> $2 = 5

slt $3, $1, $2=> $3 =0

beg $3, $0, IMemory[6]=> branch taken

lw $1, 0($0)  => $1=7

lw $2, 2($0)  => $2=5

sub $3, $1, $2 => $3 = 7-5 = 2 in binary 0000_0000_0000_0010

Sample Test 3 : DMemory[0] = 5  	DMemory[1] = 7 with Bne instruction

lw $1, 0($0) => $1 = 5

lw $2, 2($0)=> $2 = 7

slt $3, $1, $2=> $3 =1

bne $3, $0, IMemory[6]=> branch taken

lw $1, 0($0)  => $1=5

lw $2, 2($0)  => $2=7

sub $3, $1, $2 => $3 = 5-7 = -2 in binary 1111_1111_1111_1110

Sample Test 4 : DMemory[0] = 7  	DMemory[1] = 5 with Bne instruction

lw $1, 0($0) => $1 = 7

lw $2, 2($0)=> $2 = 5

slt $3, $1, $2=> $3 =0

bne $3, $0, IMemory[6]=> branch does not take

sw $1, 2($0)  => 2($0)=7

sw $2, 0($0)  => 0($0)=5

lw $1, 0($0)  => $1=5

lw $2, 2($0)  => $2=7

sub $3, $1, $2 => $3 = 5-7 = -2 in binary 1111_1111_1111_1110

Sample Test 5 : DMemory[0] = 5  	DMemory[1] = 7 with Beg instruction without noop

There are two hazards happen in this sample test. Therefore, CPU cannot compute the value for two instruction slt $3, $1, $2 and sub $3,$1,$2

| IFID_IR | IDEX_IR | EXMEM_IR | MEMWB_IR |
| --- | --- | --- | --- |
| beg $3,$0, IMemory[6] | slt $3, $1, $2 | lw $2, 2($0) | lw $1 , 0($0) |
|  | sub $3, $1, $2 | lw $2, 2($0) | lw $1, 0($0) |

The instruction slt $3,$1,$2 need value from register $1,but  $1 do not store value. Then, CPU cannot compute this instruction.

Similarly, the instruction sub $3, $1, $2 need value of registers $1, $2 from the previous instructions : lw $2, 2($0) and lw $1 , 0($0). However, the instructions is not done.

Sample Test 6 : DMemory[0] = 5  	DMemory[1] = 7 with Bne instruction without noop

Like the sample test 5. There are two hazards happen in this sample test. Therefore, CPU cannot compute the value for two instruction slt $3, $1, $2 and sub $3,$1,$2