Central Connecticut State University

BUILDING  A PIPELINED DATAPATH WITH 3-STAGES (IF,ID,EX)

Report 3

prepared by

Thi Minh Huyen Le

Brendan Manzolli

Erik Marrero

Fall - 2021

- Tasks:

Thi Minh Huyen Le: Instruction Memory, Combine Code.

Brendan Manzolli: Multiplexes, Diagrams

Erik Marrero: MainControl, Instructions

- Description

Instructions:

| Instruction (I-Type: addi) | Opcode (4 bits) [15-12] | Rs (2 bits) [11-10] | Rt (2 bits) [9-8] | Value or Address (8 bits) [7-0] | Binary Instruction |
| --- | --- | --- | --- | --- | --- |
| addi $t1,$0,15 | 0101 | 00 | 01 | 00001111 | 0101_00_01_00001111 |
| addi $t2,$0,7 | 0101 | 00 | 10 | 00000111 | 0101_00_10_00000111 |
| nop | 0000 | 00 | 00 | 00000000 | 0000_00_00_00000000 |

| Instruction (R-Type) | Opcode (4 bits) [15-12] | Rs (2 bits) [11-10] | Rt (2bit) [9-8] | Rd (2bits) [7-6] | Unused (6 bits) [5-0] | Binary Instruction |
| --- | --- | --- | --- | --- | --- | --- |
| and $t3,$t1,$t2 | 0010 | 01 | 10 | 11 | 000000 | 0010_01_10_11_000000 |
| sub $t2,$t1,$t3 | 0001 | 01 | 11 | 10 | 000000 | 0001_01_11_10_000000 |
| or $t2,$t2,$t3 | 0011 | 10 | 11 | 10 | 000000 | 0011_10_11_10_000000 |
| add $t3,$t2,$t3 | 0000 | 10 | 11 | 11 | 000000 | 0000_10_11_11_000000 |
| slt $t1,$t3,$t2 | 0100 | 11 | 10 | 01 | 000000 | 0100_11_10_01_000000 |
| slt $t1,$t2,$t3 | 0100 | 10 | 11 | 01 | 000000 | 0100_10_11_01_00000 |

- Block logic diagrams

CPU

1-bit ALU for bits 0-14

1-bit ALU for bit 15

16bit-ALU

2x1 Multiplexer

4x1 Multiplexer

The truth table for MainControl

| Instruction | OpCode | RegDst | ALUSrc | RegWrite | ALUCtl |
| --- | --- | --- | --- | --- | --- |
| add | 0000 | 1 | 0 | 1 | 010 |
| sub | 0001 | 1 | 0 | 1 | 110 |
| and | 0010 | 1 | 0 | 1 | 000 |
| or | 0011 | 1 | 0 | 1 | 001 |
| slt | 0100 | 1 | 0 | 1 | 111 |
| addi | 0101 | 0 | 1 | 1 | 010 |
| slti | 0110 | 0 | 1 | 1 | 111 |
| andi | 0111 | 0 | 1 | 1 | 000 |
| ori | 1000 | 0 | 1 | 1 | 001 |

Verilog source code

//Group : Thi Minh Huyen Le, Brendan Manzolli, Erik Marrero // Behavioral model of MIPS - single cycle implementation, R-types and addi,slti,anddi,ori // --------------REGISTER FILE --------------- // Assign RR1, RR2 to RD1, RD2 // When RegWrite is on, assign WD to WR module reg_file (RR1,RR2,WR,WD,RegWrite,RD1,RD2,clock); input [1:0] RR1,RR2,WR; input [15:0] WD; input RegWrite,clock; output [15:0] RD1,RD2; reg [15:0] Regs[0:3]; assign RD1 = Regs[RR1]; assign RD2 = Regs[RR2]; initial Regs[0] = 0; always @(negedge clock) if (RegWrite==1 & WR!=0) Regs[WR] <= WD; endmodule //-----------END REGISTER FILE----------- //--------------ALU---------------------- // A 32-bit ALU constructed from the 31 copies of the 1-bit ALU in the top and one 1-bit ALUmsb in the bottom. module ALU (op,a,b,result,zero); input [15:0] a; input [15:0] b; input [2:0] op; output [15:0] result; output zero; wire c1,c2,c3,c4,c5,c6,c7,c8,c9,c10,c11,c12,c13,c14,c15,c16; ALU1 alu0 (a[0],b[0],op[2],op[1:0],set, op[2], c1,result[0]); ALU1 alu1 (a[1],b[1],op[2],op[1:0],1'b0, c1, c2,result[1]); ALU1 alu2 (a[2],b[2],op[2],op[1:0],1'b0, c2, c3,result[2]); ALU1 alu3 (a[3],b[3],op[2],op[1:0],1'b0, c3, c4,result[3]); ALU1 alu4 (a[4],b[4],op[2],op[1:0],1'b0, c4, c5,result[4]); ALU1 alu5 (a[5],b[5],op[2],op[1:0],1'b0, c5, c6,result[5]); ALU1 alu6 (a[6],b[6],op[2],op[1:0],1'b0, c6, c7,result[6]); ALU1 alu7 (a[7],b[7],op[2],op[1:0],1'b0, c7, c8,result[7]); ALU1 alu8 (a[8],b[8],op[2],op[1:0],1'b0, c8, c9,result[8]); ALU1 alu9 (a[9],b[9],op[2],op[1:0],1'b0, c9, c10,result[9]); ALU1 alu10 (a[10],b[10],op[2],op[1:0],1'b0, c10, c11,result[10]); ALU1 alu11 (a[11],b[11],op[2],op[1:0],1'b0, c11, c12,result[11]); ALU1 alu12 (a[12],b[12],op[2],op[1:0],1'b0, c12, c13,result[12]); ALU1 alu13 (a[13],b[13],op[2],op[1:0],1'b0, c13, c14,result[13]); ALU1 alu14 (a[14],b[14],op[2],op[1:0],1'b0, c14, c15,result[14]); ALUmsb alu15 (a[15],b[15],op[2],op[1:0],1'b0, c15, c16,result[15],set); or or1(or01, result[0], result[1], result[2], result[3], result[4], result[5], result[6], result[7], result[8], result[9], result[10], result[11], result[12], result[13], result[14], result[15]); not not1(zero, or01); endmodule // 1-bit ALU for bits 0-2 // A 1-bit ALU that performs AND, OR, addition , subtraction on a and b, module ALU1 (a,b,binvert,op,less,carryin,carryout,result); input a,b,less,carryin,binvert; input [1:0] op; output carryout,result; wire sum, a_and_b, a_or_b, b_inv; not not1(b_inv, b); mux2x1 mux1(b,b_inv,binvert,b1); and and1(a_and_b, a, b); or or1(a_or_b, a, b); fulladder adder1(sum,carryout,a,b1,carryin); mux4x1 mux2(a_and_b,a_or_b,sum,less,op[1:0],result); endmodule // 1-bit ALU for the most significant bit // A 1-bit ALU that performs AND, OR, addition , subtraction on a and b, GIVE THE MOST SIGNIFICANT BIT AS AN OUTPUT SUM module ALUmsb (a,b,binvert,op,less,carryin,overflow,result,sum); input a,b,less,carryin,binvert; input [1:0] op; output overflow,result,sum; wire sum, a_and_b, a_or_b, b_inv; not not1(b_inv, b); mux2x1 mux1(b,b_inv,binvert,b1); and and1(a_and_b, a, b); or or1(a_or_b, a, b); fulladder adder1(sum,overflow,a,b1,carryin); mux4x1 mux2(a_and_b,a_or_b,sum,less,op[1:0],result); endmodule module halfadder (S,C,x,y); input x,y; output S,C; xor (S,x,y); and (C,x,y); endmodule module fulladder (S,C,x,y,z); input x,y,z; output S,C; wire S1,D1,D2; halfadder HA1 (S1,D1,x,y), HA2 (S,D2,S1,z); or g1(C,D2,D1); endmodule // Multiplexor two 1bit-inputs module mux2x1(A,B,select,OUT); input A,B,select; output OUT; wire wire0, wire1; not not1 (notS,select); and and1 (wire0, A,notS), and2 (wire1, B,select); or or1 (OUT,wire0,wire1); endmodule // Multiplexor four 1bit-inputs module mux4x1(i0,i1,i2,i3,select,y); output y; // Output input i0, i1, i2, i3; // Input ports. input [1:0] select; // Select lines. // intermediate wires wire inv0, inv1; // Inverter outputs. wire a0, a1, a2, a3; // AND gates outputs. // Inverters. not not_1 (inv1, select[1]); not not_0 (inv0, select[0]); // 4-input AND gates. and and_0 (a0, i0, inv1, inv0); and and_1 (a1, i1, inv1, select[0]); and and_2 (a2, i2, select[1], inv0); and and_3 (a3, i3, select[1], select[0]); // 4-input OR gate. or or_0 (y, a0, a1, a2, a3); endmodule //--------------END ALU----------------- //--------------MULTIPLEXORS------------- // Multiplexor two 16-bit inputs. module mux16bitx2x1 (A,B,Select,Out); input [15:0] A,B; input Select; output [15:0] Out; mux2x1 m0 (A[0],B[0],Select,Out[0]), m1 (A[1],B[1],Select,Out[1]), m2 (A[2],B[2],Select,Out[2]), m3 (A[3],B[3],Select,Out[3]), m4 (A[4],B[4],Select,Out[4]), m5 (A[5],B[5],Select,Out[5]), m6 (A[6],B[6],Select,Out[6]), m7 (A[7],B[7],Select,Out[7]), m8 (A[8],B[8],Select,Out[8]), m9 (A[9],B[9],Select,Out[9]), m10 (A[10],B[10],Select,Out[10]), m11 (A[11],B[11],Select,Out[11]), m12 (A[12],B[12],Select,Out[12]), m13 (A[13],B[13],Select,Out[13]), m14 (A[14],B[14],Select,Out[14]), m15 (A[15],B[15],Select,Out[15]); endmodule // Multiplexor two 2-bit inputs. module mux2bitx2x1 (A,B,Select,Out); input [1:0] A,B; input Select; output [1:0] Out; mux2x1 m0 (A[0],B[0],Select,Out[0]), m1 (A[1],B[1],Select,Out[1]); endmodule //--------------END MULTIPLEXORS------------ //--------------MAIN CONTROL--------------- /*** 16-bit CPU control source code ***/ module mainCtrl (op, ctrl); input [3:0] op; output reg [5:0] ctrl; // ctrl bits: RegDst, ALUSrc, RegWrite, ALUCtl(3bits) always @(op) case (op) 4'b0000: ctrl <= 6'b101010; // ADD 4'b0001: ctrl <= 6'b101110; // SUB 4'b0010: ctrl <= 6'b101000; // AND 4'b0011: ctrl <= 6'b101001; // OR 4'b0100: ctrl <= 6'b101111; // SLT 4'b0101: ctrl <= 6'b011010; // ADDI 4'b0110: ctrl <= 6'b011111; // SLTI 4'b0111: ctrl <= 6'b011000; // ANDI 4'b1000: ctrl <= 6'b011001; // ORI endcase endmodule //------------END MAIN CONTROL---------------- //------------CPU-------------- module CPU (clock,PC,IFID_IR,IDEX_IR,WD); input clock; output [15:0] PC,IFID_IR,IDEX_IR,WD; /* initial begin // Program with nop's - no hazards IMemory[0] = 16'b0101_00_01_00001111; // addi $t1, $0, 15 ($t1=15) IMemory[1] = 16'b0101_00_10_00000111; // addi $t2, $0, 7 ($t2= 7) IMemory[2] = 16'b0000_00_00_00000000; // nop IMemory[3] = 16'b0010_01_10_11_000000; // and $t3, $t1, $t2 ($t3= 7) IMemory[4] = 16'b0000_00_00_00000000; // nop IMemory[5] = 16'b0001_01_11_10_000000; // sub $t2, $t1, $t3 ($t2= 8) IMemory[6] = 16'b0000_00_00_00000000; // nop IMemory[7] = 16'b0011_10_11_10_000000; // or $t2, $t2, $t3 ($t2=15) IMemory[8] = 16'b0000_00_00_00000000; // nop IMemory[9] = 16'b0000_10_11_11_000000; // add $t3, $t2, $t3 ($t3=22) IMemory[10] = 16'b0000_00_00_00000000; // nop IMemory[11] = 16'b0100_11_10_01_000000; // slt $t1, $t3, $t2 ($t1= 0) IMemory[12] = 16'b0100_10_11_01_000000; // slt $t1, $t2, $t3 ($t1= 1) end */ initial begin // Program without nop's - wrong results due to data hazards IMemory[0] = 16'b0101_00_01_00001111; // addi $t1, $0, 15 ($t1=15) IMemory[1] = 16'b0101_00_10_00000111; // addi $t2, $0, 7 ($t2= 7) IMemory[2] = 16'b0010_01_10_11_000000; // and $t3, $t1, $t2 ($t3= 7) IMemory[3] = 16'b0001_01_11_10_000000; // sub $t2, $t1, $t3 ($t2= 8) IMemory[4] = 16'b0011_10_11_10_000000; // or $t2, $t2, $t3 ($t2=15) IMemory[5] = 16'b0000_10_11_11_000000; // add $t3, $t2, $t3 ($t3=22) IMemory[6] = 16'b0100_11_10_01_000000; // slt $t1, $t3, $t2 ($t1= 0) IMemory[7] = 16'b0100_10_11_01_000000; // slt $t1, $t2, $t3 ($t1= 1) end // Pipeline stages //=== IF STAGE === wire [15:0] NextPC; reg[15:0] PC, IMemory[0:1023]; //-------------------------------- reg[15:0] IFID_IR; //-------------------------------- ALU fetch (3'b010,PC,16'b10,NextPC,Unused); //=== ID STAGE === wire [5:0] Control; wire [15:0] RD1,RD2,SignExtend,WD; wire [15:0] FWD_RD1,FWD_RD2; // Outputs of the forwarding muxes //---------------------------------------------------- reg [15:0] IDEX_IR; // For monitoring the pipeline reg IDEX_RegWrite,IDEX_ALUSrc,IDEX_RegDst; reg [15:0] IDEX_RD1,IDEX_RD2,IDEX_SignExt; reg [1:0] IDEX_rt,IDEX_rd; wire [1:0] WR; //---------------------------------------------------- reg_file rf (IFID_IR[11:10],IFID_IR[9:8],WR,WD,IDEX_RegWrite,RD1,RD2,clock); mainCtrl MainCtr (IFID_IR[15:12],Control); assign SignExtend = {{8{IFID_IR[7]}},IFID_IR[7:0]}; //=== EXE STAGE === wire [15:0] B,ALUOut; reg [2:0] ALUctl; reg condition; ALU ex (ALUctl, IDEX_RD1, B, ALUOut, Zero); mux16bitx2x1 muxB (IDEX_RD2, IDEX_SignExt, IDEX_ALUSrc, B); // ALUSrc Mux mux2bitx2x1 muxWR (IDEX_rt, IDEX_rd, IDEX_RegDst, WR); // RegDst Mux assign WD = ALUOut; // Forwarding multiplexers assign assign1 = (IDEX_RegWrite && WR==IFID_IR[11:10]); assign assign2 = (IDEX_RegWrite && WR==IFID_IR[9:8]); mux16bitx2x1 ReadD1 (RD1,ALUOut,assign1,FWD_RD1); mux16bitx2x1 ReadD2 (RD2,ALUOut,assign2,FWD_RD2); // assign FWD_RD1 = (IDEX_RegWrite && WR==IFID_IR[11:10]) ? ALUOut: RD1; // assign FWD_RD2 = (IDEX_RegWrite && WR==IFID_IR[9:8]) ? ALUOut: RD2; initial begin PC = 0; IFID_IR = 0; // clear pipeline register to avoid forwarding from empty pipeline IDEX_RegWrite =0; end // Running the pipeline always @(negedge clock) begin // Stage 1 - IF PC <= NextPC; IFID_IR <= IMemory[PC>>1]; // Stage 2 - ID IDEX_IR <= IFID_IR; // For monitoring the pipeline {IDEX_RegDst,IDEX_ALUSrc,IDEX_RegWrite,ALUctl} <= Control; // No Forwarding IDEX_RD1 <= RD1; IDEX_RD2 <= RD2; // Forwarding (NOTE: clear IFID_IR and IDEX_RegWrite, or add another instruction, e.g. a nop in the beginning of the program) IDEX_RD1 <= FWD_RD1; IDEX_RD2 <= FWD_RD2; IDEX_SignExt <= SignExtend; IDEX_rt <= IFID_IR[9:8]; IDEX_rd <= IFID_IR[7:6]; // Stage 3 - EX // No transfers needed here - on negedge WD is written into register WR end endmodule // Test module module test (); reg clock; wire [15:0] PC,IFID_IR,IDEX_IR,WD; CPU test_cpu(clock,PC,IFID_IR,IDEX_IR,WD); always #1 clock = ~clock; initial begin $display (" PC IFID_IR IDEX_IR WD"); $monitor ("%3d %b %b %3d", PC,IFID_IR,IDEX_IR,WD); clock = 1; #29 $finish; end endmodule /* Compiling and simulation Program with nop's ------------------------------ PC IFID_IR IDEX_IR WD 0 xxxxxxxx xxxxxxxx 0 4 2009000f xxxxxxxx 0 8 200a0007 2009000f 15 12 00000000 200a0007 7 16 012a5824 00000000 0 20 00000000 012a5824 7 24 012b5022 00000000 0 28 00000000 012b5022 8 32 014b5025 00000000 0 36 00000000 014b5025 15 40 014b5820 00000000 0 44 00000000 014b5820 22 48 016a482a 00000000 0 52 014b482a 016a482a 0 56 xxxxxxxx 014b482a 1 60 xxxxxxxx xxxxxxxx X Program without nop's ------------------------------ PC IFID_IR IDEX_IR WD 0 xxxxxxxx xxxxxxxx 0 4 2009000f xxxxxxxx 0 8 200a0007 2009000f 15 12 012a5824 200a0007 7 16 012b5022 012a5824 X 20 014b5025 012b5022 x 24 014b5820 014b5025 X 28 016a482a 014b5820 x 32 014b482a 016a482a X 36 xxxxxxxx 014b482a X 40 xxxxxxxx xxxxxxxx X 44 xxxxxxxx xxxxxxxx X 48 xxxxxxxx xxxxxxxx X 52 xxxxxxxx xxxxxxxx X 56 xxxxxxxx xxxxxxxx X 60 xxxxxxxx xxxxxxxx X */

- Test result

Test1 : Code with the instructions no-opp

Test2 : Code without the instructions no-opp

Test3 : Code without the instructions no-opp and using forwarding